|  |  | PETROVIETNAM OIL JOINT STOCK CORPORATION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PROJECT |  | PETROLEUM STORAGE TANK EXPANSION NGHI SON PHASE 2 |  |  |  |  |  |
| TECHNICAL REQUISITION DOCUMENT FOR TRD-008 MANUAL VALVE |  |  |  |  |  |  |  |
| A | 02-Mar-25 | Issue for Procurement | N.M.Huong | N.M.Huong | P.Q.Phong |  |  |
| REV.No | DATE | DESCRIPTION | PREP | REVIEW | APPROVAL | PTH-TM | PV OIL |
|  |  | JOINT VENTURE CONTRACTORS (PTSC THANH HOA/ TAM MINH) |  |  | Doc.No: TRD-008 |  |  |
|  |  |  |  |  | Page: 21 (Include Cover ) |  |  |
| This DOCUMENT is the property of PTSC Thanh Hoa Technical Services Corporation. Therefore, it shall not be released to any third party without permission of authorized personnel of the PTSC Thanh Hoa Technical Services Corporation. |  |  |  |  |  |  |  |

TABLE OF CONTENTS

1.	INTRODUCTION	3

2.	DOCUMENT PURPOSE	3

3.	LANGUAGE AND UNITS	3

4.	ITEMS AND QUANTITIES	3

5.	CONFLICTING REQUIREMENTS	3

6.	DEVIATION LIST	3

7.	SCOPE OF WORK AND SUPPLY	4

8.	GENERAL REQUIREMENTS	4

9.	INSPECTION AND TESTS	6

10.	COORDINATION MEETING	7

11.	APPLICABLE DOCUMENT	7

12.	VENDOR DATA REQUIREMENT	8

13.	DOCUMENT REQUIREMENTS	8

14.	ATTACHMENTS	8

# INTRODUCTION

###### Project name: petroleum storage tank expansion 4.900 m3

###### Client:  PV OIL

###### Contractor: PTSC THANH HOA

###### Construction location: Industrial Park No. 2, Nghi Son Economic Zone, Hai Yen Commune, Tinh Gia District, Thanh Hoa Province.

###### The total expanded capacity is 4.900m³ as follows:

###### 01 tank 4.900m³ tank DO

# DOCUMENT PURPOSE

###### This document, together with the Specifications and Data Sheets provided separately, defines the minimum requirements for design, fabrication, manufacture, painting, inspection, testing, certification, transportation preparation, supply, and commissioning for MANUAL VALVE.

###### The VENDOR shall ensure that all equipment and services supplied meet the requirements of this TRD, the Project Data Sheets, Specifications and of the Codes, Standards and Specifications referenced herein.

# LANGUAGE AND UNITS

## Language

###### All documents and drawings to be submitted shall be lettered in English language.

## Units of measurement

###### Units of measurement for all engineering documents shall be in the System International (SI).

# ITEMS AND QUANTITIES

###### Refer to MTO and datasheet.

# CONFLICTING REQUIREMENTS

###### The Vendor shall notify Purchaser of any conflicts between this TRD, specifications, the related data sheets, the Codes and Standards and any other specifications noted herein. Resolutions and/or interpretation precedence shall be obtained from Purchaser in writing before proceeding with the design/manufacture.

# DEVIATION LIST

###### The Vendor’s quotation shall be in full conformity with Purchaser’s requirements. Unless exceptions, deviations or alternatives are clearly defined and listed on the attached Purchaser's form "Deviation List" and submitted with Vendor's quotation, the requirements and conditions of this Requisition shall be deemed to be accepted by the Vendor.

# SCOPE OF WORK AND SUPPLY

## Scope of Work and Supply

###### VENDOR's scope of work and supply shall include but not be limited to design, engineering, material procurement, manufacture, inspection, testing, surface preparation & painting, packaging for shipment and delivery of the equipment (as per the required delivery terms and conditions) in accordance with the table below.

###### The VENDOR shall be responsible for the supply of item(s) in all respects for the safe and reliable operation of the package.

| No. | Q’Ty | Description |
| --- | --- | --- |
|  | Bulk | Material take off as per MTO, specification and datasheet. |
|  | 1 Lot | Engineering / Sizing Calculation |
|  | 1 Lot | Surface Preparation and Painting up to the final coat |
|  | 1 Lot | Inspection, Certification and Testing |
|  | 1 Lot | Material certificates follow EN10204 Type 3.1 |
|  | 1 Lot | - NDE/NDT, PMI - Hydrostatic test / Seat Leakage Test - Factory acceptance test (FAT) |
|  | 1 Lot | Spare parts for Installation, Commissioning and Start-up |
|  | 1 Lot | Recommended spare parts for 2 years of normal operation (if required) |
|  | 1 Lot | Documentation as per VENDOR Document Requirement List (VDRL) |
|  | 1 Lot | Export Packing, Preservation for Long Term Storage and Preparation for Shipment |
|  | 1 Lot | Transportation and other related cost to transport the Goods to the Hai Phong. |

## Exclusions

###### The following are excluded from VENDOR’s scope of work: N/A

# GENERAL REQUIREMENTS

## General

###### The Package shall be strictly in accordance with the data sheets, drawings, specifications, codes, and standards specified.

###### Compliance by the Vendor with the provision of this specification does not relieve him of his responsibility to furnish the package/ material and accessories of a proper design to meet the specified duty and / or local codes governing health and safety.

###### All applicable documents mentioned below are attached with the request for inquiry. However, if the Vendor is not in possession of any of the listed or referenced Purchaser documents, it shall be Vendor's sole responsibility to obtain them from the Purchaser. Otherwise, it shall be assumed that the Vendor has received, read and understood the total contents of the documents.

###### All materials and parts shall be new and un-used and free from defects and imperfections that would adversely affect the life or performance of the system.

###### Vendor data shall include detailed design drawings and updated data sheets of each material and appurtenances. Detail design drawings shall be fully dimensioned and complete with Bill of Materials.

###### No fabrication would commence until Purchaser has approved the drawings.

###### Purchaser's approval on drawings does not relieve Vendor of compliance with applicable codes, specifications, safety design, or meeting the requirements of governmental agencies.

###### Any work and documentation related to facilitating design appraisal approvals and inspection from Purchaser appointed Certification Authority (CA) and QA/QC Inspector (TPI) shall also be included in Vendor’s scope of supply

## Design Life, Availability and Reliability

###### The design life of the facility and all associated equipment, components and systems is minimum 30 years.

###### All components which for practical, safety or cost-efficiency reasons are unable to meet the required design life shall be identified as soon as possible. Their expected service life shall be informed to Purchaser and provision made in the system design for maintenance to extend component life or routine change-out.

## Preparation for Shipment

###### Equipment shall be prepared for export shipment and shall be created to provide maximum protection during shipment and extended outdoor storage.

###### Any equipment liable to be damaged during shipment shall be disassembled, packed separately, shipped with the unit and reassembled onsite under the supervision of Vendor’s commissioning/start-up technician.

###### Each crate shall be clearly identified in accordance with the customer's specification. As a minimum the crate shall be marked with the Purchase Order number, the item number, the shipping mass and the delivery address.

###### Flanged process connections shall be protected by steel cover plates bolted in place with at least 4 bolts then sealed with tape. All nozzles and branches shall be protected against damage during transportation and erection.

###### The equipment shall be suitably preserved to maintain in good condition throughout the phases of storage until ready to be operated.

###### Threaded nozzle connections shall be protected with threaded plugs. However, telltale holes in reinforcing pads shall be protected with non-hardening plastic sealant or packed with grease.

###### Exposed machined surfaces, including threads extending beyond the nuts, shall be protected with an easily removable rust-preventive coating.

###### All equipment shall be suitably protected for shipment and securely anchored during transit. Separate or loose parts shall be completely boxed with identification tag.

###### Tools and spare parts shall be packed in metal boxes suitable for long term storage. The boxes shall be galvanized or coated in accordance with Project requirement.

## Guarantee and Warranty

###### The Vendor shall have final and total responsibility for the design and performance of all material supplied under this specification.

###### The Vendor shall replace and install without cost to the Purchaser any materials, supplies which fail under design conditions due to defects in material or workmanship if the defect is observed and/or such failure occurs within the guarantee/warranty period. Acceptance of this order will signify acceptance of all conditions of this guarantee.

###### The guarantee period shall be 12 months from commercial operating date.

## Spare part

### Start-up and Commissioning Spares

###### The Vendor shall include, as part of his bid, a detailed, price list of start-up and commissioning spares which shall be supplied as part of the original purchase package.

### Two Year Spares

###### The Vendor shall include, as part of his bid, a detailed, list of recommended spare parts for two years continuous operation. These spares shall be costed for separate purchase.

# INSPECTION AND TESTS

###### Inspection and tests shall be made by the VENDOR in accordance with:

###### [X] Test & Inspection should be carried out acc. to the above applicable codes and standards

###### [X] Maker’s standard plan & procedure as approved by PURCHASER.

###### [X] Local regulation, if any

###### The OWNER and/or Purchase inspector shall hold a right to be present at any equipment and/or materials manufacturing stage.

# COORDINATION MEETING

###### After the purchase order is placed, VENDOR shall, upon receipt of PURCHASER’s notice, send a sufficient number of qualified personnel at VENDOR’s cost to hold the meetings according to the following schedule:

| ###### [ x ] Kick-off meeting | ###### Two (2) week(s) after the order | ###### [ ] Vendor office | ###### [ x ] Purchaser’s office |
| --- | --- | --- | --- |
| ###### [ x ] Coordination meeting | ###### * Note 11amonth(s) after the order | ###### [ ] Vendor office | ###### [ ] Purchaser’s office |
| ###### [ ] Design review meeting | ###### ________month(s) after the order | ###### [ ] Vendor office | ###### [ ] Purchaser’s office |
| ###### [ x ] Pre-inspection meeting | ###### 1 month(s) before Fab. start | ###### [ ] Vendor office | ###### [ x ] Purchaser’s office |

###### * Note 11a : The time period for the same to be agreed at kick-off meeting.

# APPLICABLE DOCUMENT

###### Documents listed below shall make an integral part of this material requisition:

| No | Doc No | Document Title | Remark |
| --- | --- | --- | --- |
| 1 |  | MTO for Manual Valve |  |
| 2 |  | Specification Manual Valve |  |
| 3 |  | Datasheet Manual valve |  |
| 4 |  |  |  |

# VENDOR DATA REQUIREMENT

###### The VENDOR shall submit the completed documents and drawings, in accordance with the “Vendor Data Requirements” in time.

###### Vendor Data Requirements shall be referred to in Attachment #1.

# DOCUMENT REQUIREMENTS

###### All documents and technical data shall be in the English language. Submission shall be made under cover of a transmittal, marked for the attention of Supplier/ Vendor Data control and referencing the Company Purchase Order.

###### Documents shall be full size A1, A2, A3 and A4. A0 size is not acceptable. Legible drawings at A3 size are preferred for review. Full size drawings (A1 or A2) should be submitted as final copies.

###### Drawing prints larger than A4 shall be folded to A4 size, with the title block visible at the bottom right hand corner. Originals and transparencies shall be supplied rolled up and adequately protected. Documents larger than A3 to be included in manuals shall be inserted in pre-punched plastic sleeves.

###### Photocopy quality shall ensure that legibility and clarity of documents is maintained.

###### Manuals shall be supplied in loose-leaf hard cover binders.

# ATTACHMENTS

###### Attachment #1: Vendor Data Requirement List

###### Attachment #2: Purchaser format

###### Attachment #3: MTO for Manual Valve

###### Attachment #4: SPEC for Manual Valve

###### Attachment #5: Datasheet for Manual Valve

ATTACHMENT #1

VENDOR DATA REQUIREMENT LIST

| Item no. | VP Code | Data/Document Description | Provide with Bid | For Review | For Approval/ Construction | For Final | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| I |  | COMMERCIAL/CONTRACTUAL |  |  |  |  |  |
| 1 | C01 | VENDOR Data Requirement | E | E | E | *3 |  |
| 2 | C02 | Engineering, Procurement, Manufacturing and Test Schedule | E | E | E | *3 |  |
| 3 | C03 | Sub Contract Work Proposal |  | E | E | *3 |  |
| 4 | C04 | Delivery, Handling and Packing Proposals |  | E | E | *3 |  |
| 5 | C05 | Exception and Non-Conformity Form | E | E | E | *3 |  |
| 6 | C06 | VENDOR’s Sub-Orders (Un-Priced copy) |  |  |  |  |  |
| 7 | C07 | Warranty/Guarantee Confirmation |  | E | E | *3 |  |
| 8 | C08 | VENDOR/SUB-VENDOR List | E | E | E | *3 |  |
| 9 | C09 | Progress Reports (Weekly and Monthly) |  | E | E | *3 |  |
| 10 | C10 | Project Schedule |  | E | E | *3 |  |
| 11 | C11 | Control Procedures |  | E | E | *3 |  |
| 12 | C12 | Engineering/Procurement/Fabrication Schedule |  | E | E | *3 |  |
| 13 | C13 | Proven Design and Manufacturing Experience List | E | E | E | *3 |  |
| II |  | DRAWING - GENERAL |  |  |  |  |  |
| 14 | D01 | General Arrangement Drawings | E | E | E | *3 |  |
| 15 | D02 | Detail/Assembly/Fabrication Drawings |  | E | E | *3 |  |
| 16 | D03 | Cross Section Drawing/Exploded View Drawings |  | E | E | *3 |  |
| 17 | D04 | Internals Drawings/ Miscellaneous Drawings |  | E | E | *3 |  |
| 18 | D05 | Lifting/Transportation Drawings |  | E | E | *3 |  |
| 19 | D06 | Name Plate Drawings |  | E | E | *3 |  |
| 20 | D07 | Installation & Dismantling Drawings/Procedures (As Applicable) |  |  |  |  |  |
| 21 | D08 | System architecture |  |  |  |  |  |
| III |  | PROCESS/MECHANICAL/PIPING/STRUCTURAL |  |  |  |  |  |
| 22 | M01 | Foundation Load and Anchor Bolt Location |  |  |  |  |  |
| 23 | M02 | Equipment Design Calculations including Transportation |  | E | E | *3 |  |
| 24 | M03 | Structural Design Calculations |  | E | E | *3 |  |
| 25 | M04 | Pipe Stress Calculations |  | E | E | *3 |  |
| 26 | M05 | Process Flow Diagrams including Heat and Mass Balance |  |  |  |  |  |
| 27 | M06 | Piping and Instrument Diagrams |  | E | E | *3 |  |
| 28 | M07 | Process Sizing Calculations | E | E | E | *3 |  |
| 29 | M08 | Utility Calculations |  |  |  |  |  |
| 30 | M09 | Cause & Effect Charts |  |  |  |  |  |
| 31 | M10 | Equipment Datasheet including specialty items |  | E | E | *3 |  |
| 32 | M11 | Weight Datasheet |  | E | E | *3 |  |
| 33 | M12 | Skid/Module GA and Detail Drawings |  | E | E | *3 |  |
| 34 | M13 | Lifting Lug Details/ Calculations |  | E | E | *3 |  |
| 35 | M14 | Allowable Nozzle Load Calculations |  |  |  |  |  |
| 36 | M15 | Weight and Centre of Gravity Calculations |  |  |  |  |  |
| 37 | M16 | Lifting Equipment Calculations |  |  |  |  |  |
| 38 | M17 | Foundation Load Calculations |  |  |  |  |  |
| 39 | M18 | Miscellaneous Calculations |  |  |  |  |  |
| 40 | M19 | Skid Piping GA Drawings |  | E | E | *3 |  |
| 41 | M20 | Pipe Work Support Details |  |  |  |  |  |
| 42 | M21 | Valve List |  | E | E | *3 |  |
| 43 | M22 | Piping Isometrics |  |  |  |  |  |
| IV |  | INSTRUMENT/ELECTRICAL |  |  |  |  |  |
| 44 | J01 | Panel Layout and Details |  |  |  |  |  |
| 45 | J02 | JB/Panel Wiring Diagrams, Loop drawings |  |  |  |  |  |
| 46 | J03 | Instrument/Fire and Gas Installation Details/Hook-ups |  |  |  |  |  |
| 47 | J04 | Shutdown and Control Function Charts, Safe Charts, logic diagrams |  |  |  |  |  |
| 48 | J05 | Instrument/Electrical Data Sheets |  |  |  |  |  |
| 49 | J06 | Instrument Calculations including Safety Relief Valves, Control Valves, FE's, RO's, etc. |  |  |  |  |  |
| 50 | J07 | Serial Link Mapping List |  |  |  |  |  |
| 51 | J08 | Utility Consumption List |  |  |  |  |  |
| 52 | J09 | Functional Design Specification |  |  |  |  |  |
| 53 | J10 | Wiring Diagram |  |  |  |  |  |
| 54 | J11 | Earthing Diagram |  |  |  |  |  |
| 55 | J12 | Load Calculation |  |  |  |  |  |
| 56 | J13 | Cable schedule |  |  |  |  |  |
| 57 | J14 | Junction box schedule |  |  |  |  |  |
| 58 | J15 | Reliability & Availability Data |  |  |  |  |  |
| 59 | J16 | Copies of test certificates issued from the appropriate certifying authorities for 1) equipment to be located in hazardous areas and 2) suitable IP rating. |  |  |  |  |  |
| 60 | J17 | List of Vendor Documents and Drawings |  |  |  |  |  |
| 61 | J18 | Vendor Data Book (VDB) (2C + 2CD shall also be shipped with equipment at delivery) |  |  |  |  |  |
| 62 | J19 | Vendor Data Book (VDB) Index (Vendor data book index) |  |  |  |  |  |
| 63 | J20 | List of Instruments and System Cables (Type, Reference, Tag, Length, Routing) |  |  |  |  |  |
| 64 | J21 | Calculation Sheets of Intrinsically Safe Loops |  |  |  |  |  |
| 65 | J22 | Instrument Documentation (Training Plan and manual) |  |  |  |  |  |
| 66 | J23 | System Cabinets, Peripherals, Interconnection Block Diagrams (Including system network configuration) |  |  |  |  |  |
| 67 | J24 | Loop Diagrams (or Typical Database + I/O Database or Electronic File) (Typical mini loops from marshalling to system and complete electronic database for TPC to generate loop diagrams) |  |  |  |  |  |
| 68 | J25 | System Cards I/O Assignment |  |  |  |  |  |
| 69 | J26 | Control System Description, Operating Principles, Functional Design Specification (Including power supply, earthing, I.S guidelines) |  |  |  |  |  |
| 70 | J27 | Software Specification, Configuration Documentation (Schemes, Charts, Listing) and Manuals |  |  |  |  |  |
| 71 | J28 | Logic Diagrams (Interlock & Safety), Sequence Diagrams, Cause & Effect Charts (Functional logic diagrams) |  |  |  |  |  |
| 72 | J29 | Software Database Exchange Table |  |  |  |  |  |
| 73 | J30 | Dimension, Weight, Physical & Electrical Data of Instrument Cable |  |  |  |  |  |
| 74 | J31 | Cable List (Including interconnection cables supplied by contractor) |  |  |  |  |  |
| 75 | J32 | Bulletins and Catalogues (Including technical data sheet for each component) |  |  |  |  |  |
| 76 | J33 | Special Precautions for Handling and Erection |  |  |  |  |  |
| 77 | J34 | Recommendations for Storage prior and after Erection |  |  |  |  |  |
| 78 | J35 | Site Quality Control Plan (QCP) |  |  |  |  |  |
| 79 | J36 | Engineering, Procurement, Manufacturing and Test Schedule |  |  |  |  |  |
| 80 | J37 | Recommendations for Protection during Transportation |  |  |  |  |  |
| 81 | J38 | Packing List |  |  |  |  |  |
| 82 | J39 | Shipping Documents, Transport Authorization |  |  |  |  |  |
| 83 | J40 | N/A |  |  |  |  |  |
| V |  | GENERAL |  |  |  |  |  |
| 84 | G01 | Equipment List and Bill of Material |  | E | E | *3 |  |
| 85 | G02 | Technical Description |  | E | E | *3 |  |
| 86 | G03 | Technical Catalogues | E | E | E | *3 |  |
| 87 | G04 | Recommended Spare Parts For 2 Year Operation, Priced | E | E | E | *3 |  |
| 88 | G05 | Recommended Spare parts for Start-up & Commissioning, Priced | E | E | E | *3 |  |
| 89 | G06 | List of Special Tools, where applicable | E | E | E | *3 |  |
| 90 | G07 | Engineering, Procurement, Manufacturing and Test Schedule |  | E | E | *3 |  |
| 91 | G08 | Progress Report (monthly) including vendor procurement status (monthly) |  | E | E | *3 |  |
| 92 | G09 | Sub-Vendors and Main Suppliers' List |  | E | E | *3 |  |
| VI |  | PERFORMANCE |  |  |  |  |  |
| 93 | P01 | Test Procedure |  | E | E | *3 |  |
| 94 | P02 | Test Reports |  | E | E | *3 |  |
| 95 | P03 | QC Inspection Plan |  | E | E | *3 |  |
| 96 | P04 | Software overview, software listings, calculation |  |  |  |  |  |
| 97 | P05 | Formulas and computation definitions and listings |  |  |  |  |  |
| VII |  | QA/CERTIFICATION/REPORT |  |  |  |  |  |
| 98 | Q01 | Contract Quality Plan/ Project Quality Plan | E | E | E | *3 |  |
| 99 | Q02 | Welding Procedures & Qualification Test Reports |  |  |  |  |  |
| 100 | Q03 | Inspection and Test Plan | E | E | E | *3 |  |
| 101 | Q04 | Inspection and Test Reports |  | E | E | *3 |  |
| 102 | Q05 | Calibration Procedures & Results |  |  |  |  |  |
| 103 | Q06 | Hydro Test/Pneumatic Test Procedures |  | E | E | *3 |  |
| 104 | Q07 | Hydro Test/Pneumatic Test Reports |  | E | E | *3 |  |
| 105 | Q08 | Material Certification |  | E | E | *3 |  |
| 106 | Q09 | Calibration Certificates |  | E | E | *3 |  |
| 107 | Q10 | Hazardous Area Certification, IP Certification |  | E | E | *3 |  |
| 108 | Q11 | Surface Preparation and Paint Procedure and Report |  | E | E | *3 |  |
| 109 | Q12 | Pre-Commissioning/ Commissioning/Site Acceptance Test Procedure |  | E | E | *3 |  |
| 110 | Q13 | Pre-Commissioning/ Commissioning/Site Acceptance Test Report |  | E | E | *3 |  |
| 111 | Q14 | Factory Acceptance Test Procedure |  | E | E | *3 |  |
| 112 | Q15 | Factory Acceptance Test Report |  | E | E | *3 |  |
| 113 | Q16 | Weighing Procedure |  |  |  |  |  |
| 114 | Q17 | NDE/NDT Procedures & Reports |  | E | E | *3 |  |
| 115 | Q18 | Structural Analysis Report |  | E | E | *3 |  |
| 116 | Q19 | Stress analysis Report |  |  |  |  |  |
| 117 | Q20 | Certificate of Equipment Quantity and Performance per the Specification |  | E | E | *3 |  |
| 118 | Q21 | Inspection and testing procedures and results, including third-party inspection results of the metering runs |  | E | E | *3 |  |
| 119 | Q22 | Shipping, Unpacking & Storage Procedure |  | E | E | *3 |  |
| 120 | Q23 | Cable continuity & Resistance Test Report |  |  |  |  |  |
| 121 | Q24 | Certificate of Origin |  | E | E | *3 |  |
| 122 | Q25 | Certificate of Quantity & Quality |  | E | E | *3 |  |
| 123 | Q26 | Procedure for Erection and Installation of the Equipment |  | E | E | *3 |  |
| 124 | Q27 | Fabrication and Quality Control Plan |  | E | E | *3 |  |
| 125 | Q28 | Site Acceptance Tests Procedure (SAT) (The 'issue date' indicated is 8W before SAT date) |  | E | E | *3 |  |
| 126 | Q29 | Declaration of Conformity according to ATEX (Certificates for use of electrical apparatus in hazardous area & certificates for IS barriers) |  |  |  |  |  |
| VIII |  | MANUAL AND INDICES |  |  |  |  |  |
| 127 | R01 | Installation, Operation and Maintenance Manual (IOM) |  | E | E | *3 |  |
| 128 | R02 | Manufacturer’s Data Record (MDR) |  | E | E | *3 |  |
| 129 | R03 | Training Manuals (Operation and Maintenance) |  | E | E | *3 |  |
| IX |  | MISCELLANEOUS |  |  |  |  |  |
| 130 | TBA | TBA |  |  |  |  |  |

Legend:

*1 : 3C + E		*2 : 3C + E		*3 : 1OR + 5C + E + 2CD

E : Electronic file	OR : Original		C : Copy			CD : CD-ROM

ATTACHMENT #2

PURCHASER FORMAT

DEVIATION FROM PURCHASER’S SPECIFICATION

| Spec. No. | Paragraph No. | Deviation | PTSC Justification | Client Conclusion | Remarks | Cost Effect When Deviation is Applied |
| --- | --- | --- | --- | --- | --- | --- |

SPARE PARTS LIST FOR INSTALLATION, COMMISSIONING

| Item no. | Part name | Drawing no. | Part no. | Spec (material) | Installed Q’ty per unit | Recommended Q’ty | Sketch | Unit price | Total price |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

SPARE PARTS LIST FOR 2-YEAR OPERATION

| Item no. | Part name | Drawing no. | Part no. | Spec (material) | Installed Q’ty per unit | Recommended Q’ty | Sketch | Unit price | Total price |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

SPECIAL TOOL LIST

| No. | Tool name | Q’ty | Sketch | Unit price | Total price |
| --- | --- | --- | --- | --- | --- |

ATTACHMENT #3

MATERIAL TAKE OFF FOR MANUAL VALVE

ATTACHMENT #4

SPECIFICATION FOR MANUAL VALVE

ATTACHMENT #5

SPECIFICA FOR MANUAL VALVE